# Networking Basics - SS Notes

In this series of lectures, we get our networking basics right. We look at basic networking concepts like switching, routing, gateways, etc.

We then understand DNS. We look at how to configure DNS settings on a Linux system. So let's get started.

So what is a network? We have two computers, A and B, laptops, desktops, VMs on the cloud, wherever. How does system A reach B?

We connect them to a switch, and the switch creates a network containing the two systems. To connect them to a switch, we need an interface on each

host, physical or virtual, depending on the host. To see the interfaces for the host, we use the IP link command.

In this case, we look at the interface named eth0 that we will be using to connect to the switch.

Let's assume it's a network with the address 192.168.1.0. We then assign the systems IP addresses on the same network.

For this, we use the command IP addr. Once the links are up, and the IP addresses are assigned, the

computers can now communicate with each other through the switch. The switch can only enable communication within a network, which means it

can receive packets from a host on the network and deliver it to other systems within the same network. Say we have another network containing systems

C and D at address 192.168.2.0. The systems have IP addresses 192.168.2.10 and 192.168.2.11 respectively. How does a system in one network

reach a system in the other? How does system B with the IP 192.168.1.11 reach system C with the IP 192.168.2.10 on the other network?

That's where a router comes in. A router helps connect two networks together. It is an intelligent device. So think of it as another server with many network ports.

Since it connects to the two separate networks, it gets two IPs assigned, one on each network. In the first network, we assign it an IP address 192.168.1.1.

And in the second, we assign it an IP 192.168.2.1. Now we have a router connected to the two networks

that can enable communication between them. When system B tries to send a packet to system C, how does it know where the router is on the

network to send the packet through? The router is just another device on the network. There could be many other such devices.

That's where we configure the systems with a gateway or a route. If the network was a room, the gateway is a door to

the outside world, to the other networks or to the Internet. Systems need to know where that door is to go through that.

To see the existing routing configuration on a system, run the route command. It displays the kernel's routing table. And within that, as you can see, there are

no routing configurations as of now. So in this condition, your system B will not be able to reach system C.

It can only reach other systems within the same network in the range 192.168.1.0. To configure a gateway on system B to reach the systems on

network 2.0, run the IP route add command and specify that you can reach the 192.168.2.0 network through the router or gateway at 192.168.1.1.

Running the route command again shows that we have a route added to reach the 192.168.2.0 slash twenty-four network through the router.

Now remember, this has to be configured on all the systems. For example, if the system C is to send a packet to

system B, then you need to add a route on system C's routing table to access the network at one dot zero through the

router configured with the IP address two dot one. Suppose these systems need access to the internet. Say they need access to Google at 172.217.194.0 network on the internet.

So you connect the router to the internet, and then add a new route in your routing table to route all traffic to the

network one seventy-two dot two seventeen dot one ninety-four through your router. There are so many different sites on different networks on the internet.

Instead of adding a routing table entry for this same router's IP address. For each of those networks, you can simply say, for any network

that you don't know a route to, use this router as the default gateway. This way, any request to any network outside of your existing

network goes to this particular router. So in a simple setup like this, all you need is a single routing table entry with a default gateway

set to the router's IP address. Remember, instead of the word default, you could also say zero point zero point zero point zero.

It means any IP destination. Both of these lines mean the same thing, as the zero point zero point zero point zero entry in the gateway field indicates

that you don't need a gateway. For example, in this case for system C, to access any devices in the 192.168.2.0 network, it doesn't need a gateway because

it is in its own network. But say you have multiple routers in your network, one for the Internet. Another for the internal private network, then you will need to have

two separate entries, one for each network. One entry for the internal private network, and another entry with the default gateway for all other networks, including public networks.

So if you're having issues reaching the internet from your systems, this routing table and the default gateway configuration is a good place to start.

Let us now look at how we can set up a Linux host as a router. Let's start with a simple setup.

I have three hosts, A, B, and C. A and B are connected to a network, 192.168.1, and B and C to another on 192.168.2.

So host B is connected to both the networks using two interfaces eth0 and eth1. A has IP 192.168.1.5, C has 192.168.2.5, and B has an IP

on both the networks, 1.6 and 2.6. How do we get A to talk to C? But basically, if I try to ping to 192.168.2.5 from A,

it would say network is unreachable. And by now we know why that is. Host A has no idea how to reach a network at 192.168.2.

We need to tell Host A that the door or gateway to network two is through Host B. And we do that by adding a routing table entry.

We add a route to access network 192.168.2 through the gateway 192.168.1.6. If the packets were to get through to host C, host C

will have to send back responses to host A. When host C tries to reach host A at the 192.168.1 network,

it would face the same issue. So we need to let host C know that it can reach host A through host B, which is acting as a router.

So we add a similar entry into host C's routing table. This time we say to reach network 192.168.1.0, talk to host B at 192.168.2.6.

When we try to ping now, we no longer get the network unreachable error message. That means our routing entries are right.

But we still don't get any response back. By default, in Linux, packets are not forwarded from one interface to the next.

For example, packets received on eth0 on host B are not forwarded to elsewhere through eth1. This is this way for security reasons.

For example, if you had eth0 connected to your private network, and eth1 to a public network, we don't want anyone from the public

network to easily send messages to the private network, unless you explicitly allow that. But in this case, since we know that both are private networks,

and it is safe to enable communication between them, we can allow host B to forward packets from one network to the other.

Whether a host can forward packets between interfaces is governed by a setting in this system file proc/sys/net/ipv4, IP underscore forward.

By default, the value in this file is set to zero, meaning no forwarding. Set this to one and you should see the pings go through.

Now remember, simply setting this value does not persist the changes across reboots. For that, you must modify the same value in the /etc/sysctl.conf file.

So let's take away some key commands from this lecture. These will be handy in the upcoming lectures. IP link is to list and modify interfaces on the host.

IP addr command is to see the IP addresses assigned to those interfaces. IP addr add command is used to

set IP addresses on the interfaces. Now remember, changes made using these commands are only valid till restart. If you want to persist these changes, you must

set them in the /etc/network/interfaces file. IP route, or simply the route command, is used to view the routing table.

And IP route add command is used to add entries into the routing table. And finally, remember the command to check if IP forwarding is enabled

on a host, if you're working with a host configured as a router. Well, that's it for this lecture. In the next lecture, we will discuss DNS configurations.